# MAD LABS – Kids Ride-On 24 V Control System

## Architecture & Safety Framework

Project: Kids Ride-On 24 V 4WD

Maintainer: MAD Labs / MAD Manufacturing

Baseline Version: v4.0

Status: Active / Living Document

## 1. Purpose of This Document

This file defines the high-level architecture, responsibilities, and safety philosophy of the MAD Labs Kids Ride-On control system.

It is intentionally implementation-agnostic: - No firmware code - No low-level wiring diagrams - No PCB schematics

Those details live elsewhere (pinout docs, KiCad, firmware repos).

This document answers: - What each subsystem does - How power and control are structured - Where safety decisions are made - Why the system is designed this way

This is a living, append-only framework. New safety features or behavioral rules are added as sections, not replacements.

## 2. Power Architecture

- 24 V Battery Input

  - Primary system power

  - Routed directly to all motor drivers

- Logic Power (5 V)

  - Derived from 24 V via buck converter

  - Supplies:

    - Teensy 4.1

    - ESP32-S3

    - Motor driver logic pins

- Separation of Domains

  - High-current motor power remains isolated from logic control

  - Logic ground is common-reference only

## 3. Controller Roles & Separation of Concerns

### 3.1 Teensy 4.1 — Real-Time Control Controller

The Teensy 4.1 is the authoritative control brain of the vehicle.

Responsibilities: - All PWM motor control - All sensor acquisition - All safety logic - Closed-loop steering control - Soft-start / soft-stop behavior - Speed vs steering angle limiting - Fault detection and response

All sensors connect directly to the Teensy.

No safety-critical decision depends on the ESP32.

### 3.2 ESP32-S3 — OTA & Monitoring Hub

The ESP32-S3 exists to remove the need for physical access to the system.

Responsibilities: - Over-the-air firmware updates (ESP32 + Teensy) - High-speed serial monitoring - Web / OTA hub connectivity - System status indication (RGB)

The ESP32 does not: - Generate PWM - Read safety sensors - Make motion decisions

A failure of the ESP32 does not prevent safe vehicle operation.

## 4. Inter-Processor Communication

- Two dedicated UART links between Teensy and ESP32

  - UART A: Teensy firmware flashing

  - UART B: High-speed telemetry / logging

- ESP32 acts as:

  - Uploader

  - Bridge

  - Monitor

## 5. Motor Control Architecture

Three independent motor groups are controlled via BTS7960 drivers:

### 5.1 Steering Motor

- Electrically actuates steering

- Closed-loop control using magnetic encoder feedback

- Software-defined soft limits

### 5.2 Front Drive Motors

- Both front wheels driven together

- Current sensing monitored

- PWM speed control

### 5.3 Rear Drive Motors

- Both rear wheels driven together

- Current sensing monitored

- PWM speed control

All motor groups: - Use PWM for speed - Use enable pins for safety gating - Are monitored for overcurrent conditions

## 6. Sensors & User Inputs

### 6.1 Magnetic Steering Encoder

- Absolute angle measurement

- Used for:

  - Closed-loop steering

  - Steering angle limits

  - Speed to Steering Angle-based safety reduction

### 6.2 Throttle Input

- Analog potentiometer

- Represents user-requested speed

### 6.3 Steering Input

- Analog potentiometer

- User-requested steering direction

### 6.4 Shifter

- Neutral / Forward / Reverse

- Neutral enforced during fault conditions

### 6.5 Mode / E-Brake (Reserved)

- Future expansion input

- Intended for advanced safety or braking modes.. but this is not actually implemented in code and can be used for anything you desire ie does not have to be for e-brake or mode

### 6.6 Remote Control (Parental Override)

- Allows adult takeover of vehicle control

Takeover Procedure: 1. Hold Down + Reverse for 3 seconds 2. Return controls to neutral 3. Remote gains control 4. Control auto-releases after 20 seconds of inactivity

## 7. Motion Behavior & Safety Logic

### 7.1 Soft Start / Soft Stop

- Acceleration is ramped

- Deceleration is ramped

- Prevents sudden jolts

- Reduces drivetrain stress

- Must include during Takeover Procedure(prevents sudden stop on takeover if vehicle is moving fast)

### 7.2 Steering Angle vs Speed Limiting

- Maximum steering angle decreases as speed increases

- If a sharp steering angle is requested:

  - Vehicle speed is reduced first

  - Steering is then allowed

This reduces rollover risk but does not eliminate it.

## 8. System Status Indicator

### RGB Indicator (ESP32 GPIO38)

- Fiber-optic RGB output

- Always illuminated

Indicator States: - RED (Default) - ESP32 not connected - Teensy not responding - OTA server unavailable

- GREEN pulsing

  - ESP32 online

  - Teensy responding

  - OTA server reachable

- BLUE pulsing

  - OTA Updating in Progress

This provides an immediate visual health indicator.

## 9. Connector Philosophy (High Level)

- Motor drivers use multi-pin locking connectors

- Sensors use grouped, keyed connectors

- Future expansion ports are reserved and documented

Exact pin mappings are defined in the PCB pinout documentation.

## 10. Failure Modes & Responses (Why This Exists)

This section is intentionally high-level.

It does not list every fault condition.

Instead, it defines the rule:

Any detected fault must move the system to a safer state than it was previously in.

Examples: - Loss of Steering1 or angle sense feedback → cut Throttle1 - Overcurrent detected → reduce PWM or disable output - Invalid input state → force neutral

As new safety behaviors are added, they are appended here as subsections.

## 11. Living Document Policy

- This file is versioned in GitHub

- New features append sections

- Existing sections are not rewritten without version notes

This creates a clear historical record of why the system behaves the way it does.

End of Baseline Framework (v4.0)