Kids Ride-On V3 – Teensy 4.1 & ESP32-S3 Pinout

Teensy 4.1 Pinout

Analog Inputs (0–3.3V)

Pin 17 (A3) – Steering Pot (S1_WIPER)

Pin 23 (A9) – Throttle Pot (T1_WIPER)

Pin 16 (A2) – AS5600 Analog Out

Pin 24 (A10) – Steer Motor Current L

Pin 25 (A11) – Steer Motor Current R

Pin 26 (A12) – Front Drive Current L

Pin 27 (A13) – Front Drive Current R

Pin 38 (A14) – Rear Drive Current L

Pin 39 (A15) – Rear Drive Current R

Digital Inputs (3.3V Logic)

Pin 28 – Shifter Forward

Pin 29 – Shifter Reverse

Pin 30 – Mode Select

Pin 31 – E-Brake Input

PWM Outputs (3.3V)

Pin 2 – Steering LPWM

Pin 3 – Steering RPWM

Pin 6 – Front Drive LPWM

Pin 9 – Front Drive RPWM

Pin 12 – Rear Drive LPWM

Pin 13 – Rear Drive RPWM

Motor Driver Enable Outputs (3.3V → 5V)

Pin 4 – Steering L_EN

Pin 5 – Steering R_EN

Pin 10 – Front Drive L_EN

Pin 11 – Front Drive R_EN

Pin 32 – Rear Drive L_EN

Pin 33 – Rear Drive R_EN

I2C

Pin 18 – SDA

Pin 19 – SCL

UART Links to ESP32-S3

Pin 8 (TX2) → ESP32 GPIO44

Pin 7 (RX2) ← ESP32 GPIO43

Pin 14 (TX3) → ESP32 GPIO18

Pin 15 (RX3) ← ESP32 GPIO17

ESP32-S3 Used Pins

GPIO44 – UART RX from Teensy TX2

GPIO43 – UART TX to Teensy RX2

GPIO18 – UART RX from Teensy TX3

GPIO17 – UART TX to Teensy RX3

GPIO38 – RGB / Fiber Optic Status Indicator

Remote Control Pins 5V

GPIO 20 – Steering2

GPIO 21 –Throttle2