# Teensy 4.1 – Ride-On V3 PCB Pinout (V3, Updated)

# Changes from Version 2 → Version 3

• All 6 motor driver current-sense pins included under each BTS7960 motor driver (total 8 pins per driver) ALL use voltage divider with zener diode for further protection

• Removed old rotary encoder and replaced with AS5600 magnetic sensor.

• ESP32-S3 DevKitC-1 N8R8 header added with USB facing board edge for enclosure design.

• Dual-UART design added (Serial2 + Serial3 to ESP32-S3).

• Added 3‑pin header for Shifter (FWD/REV/GND) and 3 Pin Header Mode + E‑Brake (GND/MODE/E_BRAKE).

• PWM outputs confirmed on pins 2,3,6,9,12,13 with NO level shifting required.

• Analog inputs confirmed for S1, T1, AS5600 AOUT, and (all 6 current-sense lines CD74HC4050 DOWN-SHIFT).

• ENABLE pins (L_EN, R_EN) on all BTS7960 motor drivers REQUIRE 3.3V → 5V upshift via SN74AHCT244.

## Steering Potentiometer – S1 (3.3V, Direct to Teensy)

• 3.3V → S1_VCC

• Pin 17 (A3) → S1_WIPER (analog 0–3.3V, no shifter)

• GND → S1_GND

## Throttle Potentiometer – T1 (3.3V, Direct to Teensy)

• 3.3V → T1_VCC

• Pin 23 (A9) → T1_WIPER (analog, no shifter)

• GND → T1_GND

## Remote Controller PWM Steering/Throttle – S2/T2 (5V PWM → Teensy via Down-Shifter)

• 5V → VCC

• Pin 20 → S2_PWM (5V → 3.3V via CD74HC4050 DOWN-SHIFT)

• Pin 21 → T2_PWM (5V → 3.3V via CD74HC4050 DOWN-SHIFT)

• GND → GND

## AS5600 Angle Sensor – Required Analog + I²C (3.3V Native) OLD 5 Pin Header

• 3.3V → VCC (was 5V before)

• GND → GND

• Pin 18 → SDA (3.3V I²C)

• Pin 19 → SCL (3.3V I²C)

• Pin 16 (A2) → AOUT (analog 0–3.3V)

## Shifter (3.3V Logic Inputs) 3 Pin Header

• Pin 28 → SHIFTER_FWD

• Pin 29 → SHIFTER_REV

• GND → SHIFTER_GND

## Mode + E-Brake Selector (3.3V Logic) 3 Pin Header

• GND → MODE_GND

• Pin 30 → MODE_SELECT

• Pin 31 → E_BRAKE_IN

## STEER Motor Driver – BTS7960 Logic Header (8 Pins Total) 8 pin Header

• +5V → STEER_VCC

• GND → STEER_GND

• Pin 2 → STEER_LPWM (3.3V OK, no shifter)

• Pin 3 → STEER_RPWM (3.3V OK, no shifter)

• Pin 4 → STEER_L_EN (3.3V → 5V via SN74AHCT244 UPSHIFT)

• Pin 5 → STEER_R_EN (3.3V → 5V via SN74AHCT244 UPSHIFT)

• Pin 24 (A10) → STEER_IS_L (0–5V → 3.3V via voltage divider with zener diode for further protection

• Pin 25 (A11) → STEER_IS_R (0–5V → 3.3V via voltage divider with zener diode for further protection

## FRONT DRIVE Motor Driver – BTS7960 Logic Header (8 Pins Total) 8 pin Header

• +5V → FRONT_VCC

• GND → FRONT_GND

• Pin 6 → FRONT_LPWM (3.3V OK)

• Pin 9 → FRONT_RPWM (3.3V OK)

• Pin 10 → FRONT_L_EN (3.3V → 5V via SN74AHCT244 UPSHIFT)

• Pin 11 → FRONT_R_EN (3.3V → 5V via SN74AHCT244 UPSHIFT)

• Pin 26 (A12) → FRONT_IS_L (0–5V → 3.3V via voltage divider with zener diode for further protection

• Pin 27 (A13) → FRONT_IS_R (0–5V → 3.3V via voltage divider with zener diode for further protection

## REAR DRIVE Motor Driver – BTS7960 Logic Header (8 Pins Total) 8 pin Header

• +5V → REAR_VCC

• GND → REAR_GND

• Pin 12 → REAR_LPWM (3.3V OK)

• Pin 13 → REAR_RPWM (3.3V OK)

• Pin 32 → REAR_L_EN (3.3V → 5V via SN74AHCT244 UPSHIFT)

• Pin 33 → REAR_R_EN (3.3V → 5V via SN74AHCT244 UPSHIFT)

• Pin 38 (A14) → REAR_IS_L (0–5V → 3.3V via voltage divider with zener diode for further protection

• Pin 39 (A15) → REAR_IS_R (0–5V → 3.3V via voltage divider with zener diode for further protection

## Dual-UART ESP32-S3 DevKitC-1 N8R8 – UART Links (3.3V Native) 4 Pin Header

• Teensy TX2 (Pin 8) → ESP32 RX0 (GPIO44)

• Teensy RX2 (Pin 7) → ESP32 TX0 (GPIO43)

• Teensy TX3 (Pin 14) → ESP32 GPIO18

• Teensy RX3 (Pin 15) → ESP32 GPIO17

## Logic Conversion Summary

• DOWN-SHIFT (5V → 3.3V): CD74HC4050 (Remote Controller PWM + all CURRENT SENSE lines). 8x Lines

• UP-SHIFT (3.3V → 5V): SN74AHCT244 (ALL ENABLE PINS on motor drivers). 6x Lines

• PWM outputs to BTS7960 DO NOT require upshift.

*CHANGED FROM 2x UPSHIFT SN74AHCT244 to 1X

*CHANGED FROM 1x CD74HC4050 TO 2X

# Connector Standard

All PCB wire-to-board headers shall use **JST-VH Series (3.96 mm pitch) locking connectors**, rated for 18–22 AWG wire, providing positive latch retention suitable for vibration environments.

3x 8 pin  JST-VH Series (3.96 mm pitch) locking connectors

1x 5 pin JST-VH Series (3.96 mm pitch) locking connectors

1x 4 pin JST-VH Series (3.96 mm pitch) locking connectors

4x 3 pin JST-VH Series (3.96 mm pitch) locking connectors